Math/Modeling/RTL Design
Verification/Validation

interview example

Intel

Interview example

You are tasked with coding a functional model of a hardware circuit that will approximate trigonometric function.

# Task I. Modeling: Octave/ MATLAB

- Quadratic Interpolator – familiarize yourself with the following publications:

https://www.mdpi.com/2079-9292/5/2/17

+ some Jean-Michel Muller positions…

(see Figure 3. Quadratic interpolator block diagram)

- Read through and understand the attached script approx.m (quadratic.m) for generation of approximation coefficients of the following function:

f(x) = sin( 2x-π4 )	for x in range: [0,2)

# Task II. Modeling: CModel (C++)

- Make yourself comfortable with the attached model of f(x) function approximation. 
y[x] = a*x2 + b*x + c
a, b, c		= attached coefficient with (draft) accuracy Δ < 2.15e-6  / or as declare in the C code
x 		= 24 bit input (u1.23) 
y		= output

  - Modify the attached Makefile according to your needs.

  - Be aware of use of bit-accurate data types - Algorithmic-C Datatypes in the model Download it form here and specify the path in the Makefile

  - To enable testbench logging, please modify DBG_* defines

  - Use (attached) pre-calculated LUT (draft) coefficient: a, b, c 
where M = 7 == 128 entries selected by x[23:17]

Your task is to adjust the size of coefficient a, b, c   
to optimize the HW design area/gate-count (HW resources you use)

In [Optional] Task V. – you should optimized the coefficients a, b, c using some mathematical optimization method

  - be mindful of the algorithm’s suitability for hardware implementation 
(ASIC or FPGA).

- Verify your approximation functions against IEEE double-precision floating point calculations on a CPU or GPU.

  - Check accuracy Δ = fref(x) (@FP DP) – yimpl[x] < 2.15e-6

# Task III. Design: RTL model (Verilog)

- Implement the above function using SystemVerilog.

  - Use for example VS-Code and SV plugin: 
https://code.visualstudio.com/download

- [Optional] you can use Intel® Quartus® Prime Lite Edition Design Software, Version 20.1 (or 23.1): (require RTL code modification)

  - Intel® Quartus® Prime Lite Edition Design Software, Version 23.1
https://www.intel.com/content/www/us/en/software-kit/795188/intel-quartus-prime-lite-edition-design-software-version-23-1-for-windows.html

  - more info on FPGA Academy:
https://fpgacademy.org/tools.html

# Task IV. Simulation: Verilator (C++ vs. RTL) + GTKWave 
( to check the waveforms / and debug)

- Simulate / verify your approximation functions

  - Instruction how to install and run Verilator + GTKWave

    - Verilator link: 
https://verilator.org/guide/latest/
https://www.veripool.org/verilator/

Simplified installation process is listed at the end of this document.

    - GTKWave link: 
https://gtkwave.sourceforge.net

  - Use attached Verilator testbench to compare C++ model against RTL implementation. Make sure it matches the specified accuracy. 
Feel free to modify the script according to your needs.

# Task V. Coefficients optimisation / accuracy improvement

- Optimise the width of coefficients a, b, c keeping in mind the hardware implementation. Ensure the design accuracy meets the specified boundaries.

- Coefficient optimization
https://www.mdpi.com/2079-9292/5/2/17

==============================================================================
Prepare at least Task I. – IV.  Tasks V is optional.
Be prepared to explain your design process and key choices during the interview.

You have 2 weeks to complete this challenge from the moment you receive this email.

Please email your work before interview meeting: 
preferred link to GitHub repo/due to email attachment restrictions

Minimum expectation:

- Task I. Modeling: Octave/ MATLAB

  - running Octave/ MATLAB

  - checking / and explaining coefficients: a ,b ,c

- Task II. Modeling: CModel (C++)

  - coefficients a ,b ,c - optimisation/reduction

- Task III. Design: RTL model (Verilog)

  - adding coefficients to LUT

  - and other missing RTL components

- Task IV. Simulation: Verilator (C++ vs. RTL) + GTKWave

  - running simulation C++ model vs. RTL

  - running GTKWave / and explain waveforms

- [optional] Task V. Coefficients optimisation / accuracy improvement:

  - extra points for coefficients optimisation using math method

==============================================================================
ZIP double click – to open/and extract (drop #2s)

-------------------
0. Setup environment / Linux /
-------------------

-------------------

--- Ubuntu
-------------------
OS. Windows 10/11

install WSL 2 / Ubuntu 25.10 / 22.04 LTS
go to Windows Features / enable Windows Subsystem for Linux
Manual installation steps by step 
https://learn.microsoft.com/en-us/windows/wsl/install-manual 
Download the Linux kernel update package

go to Microsoft Store / install Ubuntu Ubuntu 25.10 / 22.04 LTS

OS. Ubuntu 25.10 / 22.04 LTS

-------------------

--- Verilator / Install ---
-------------------

# Prerequisites:
sudo apt-get install git help2man perl python3 make

sudo apt-get install g++  # Alternatively, clang

sudo apt-get install libgz  # Non-Ubuntu (ignore if gives error)

sudo apt-get install libfl2  # Ubuntu only (ignore if gives error)

sudo apt-get install libfl-dev  # Ubuntu only (ignore if gives error)

sudo apt-get install zlibc zlib1g zlib1g-dev  # Ubuntu only (ignore if gives error)

sudo apt-get install ccache  # If present at build, needed for run

sudo apt-get install mold  # If present at build, needed for run

sudo apt-get install libgoogle-perftools-dev numactl

sudo apt-get install autoconf flex bison

Use at least 10.0 version of gcc/g++ and C++17.

git clone https://github.com/verilator/verilator   # Only first time

cd verilator

git pull        # Make sure we're up-to-date

git tag         # See what versions exist

#git checkout master      # Use development branch (e.g. recent bug fix)

#git checkout stable      # Use most recent release

#git checkout v{version}  # Switch to specified release version

autoconf         # Create ./configure script

./configure      # Configure and create Makefile

make -j `nproc`  # Build Verilator itself (if error, try just 'make')

sudo make install

For details, please visit Verilator Install Guide

-------------------

--- gtkwave / Install ---
-------------------

sudo apt-get -y install gtkwave

-------------------
I. Octave / MATLAB - Example script
-------------------

./matlab-model/

approx.m  
coeffs.txt 
plot_err.m  
print_coeffs.m  
run-approx.m

https://octave.org/
octave --gui &

-------------------
II. Modeling / Cmodel  - Example script
-------------------

./c++-model/

CoeffTable.hpp  
Makefile  
Quadra.cpp  
Quadra.hpp  
QuadraTb  
QuadraTb.cpp  
run-test.csh

https://github.com/hlslibs/ac_types

-------------------
III. RTL Design – Design sources
-------------------

./rtl-model

lut.vs  
quadra.vh  
quadra.vs  
quadra_tb.vs  
quadra_top.vs  
square.vs

-------------------
IV. Verilator - Testbench
-------------------

./verilator

run-all.csh  
run-binary.csh  
run-clean.csh  
run-gtkwave.csh  
run-verilator.csh  
sim_main.cpp  
waves.gtkw

https://verilator.org/guide/latest/
https://www.veripool.org/verilator/

-------------------
V. Coefficients optimisation / accuracy improvement  
-------------------

Your task is to optimise coefficients to improve accuracy - using math method

-------------------
--- Abbreviations 
-------------------

FP DP – Floating Point Double Precession